MERIDIAN NORSTAR ADMIN SECTION SYSTEM SPEED DIAL

INSTRUCTIONS FOR PROGRAMMING SYSTEM SHORT CODE NUMBERS

NOTE!

DO NOT PICK UP THE HANDSET DURING PROGRAMMING. ALSO THE PHONE

WILL NOT BE ABLE TO MAKE OR RECEIVE ANY CALLS.

USING A M7310 OR M7324 TERMINAL              DISPLAY

PRESS THE "FEATURE" BUTTON

ENTER  ** 23646 (SPELLS ADMIN)          Password:

RETRY

ENTER 23646 (OR OWN PASSWORD)           1.Sys speed dial

PRESS SHOW                              Speed dial :_

PRESS NEXT OR ENTER 1st SPEED           Speed dial :**

DIAL NUMBER ** i.e. No'S 01 TO 70

PRESS SHOW                              No number stored

(STORED NUMBER COULD BE LISTED)                     CHANGE

PRESS CHANGE                            **:

AND ENTER FULL TELEPHONE NUMBER                BKSP   OK

PRESS OK                                **:

CHANGE

PRESS NEXT                              Use prime line

CHANGE

PRESS NEXT                              Display digits:Y

CHANGE

PRESS CHANGE TO "N" IF A NAME IS TO

BE DISPLAYED INSTEAD OF ANY DIGITS.

PRESS NEXT                              Name

PRESS SHOW                              Sys Spd Dial **

CHANGE

PRESS CHANGE AND ENTER NAME USING

THE KEYPAD (UP TO 16 CHARACTERS) &      <---  BKSP  --->

PRESS ARROW ON RIGHT OR "GATE" TO

MOVE THE CURSOR TO THE NEXT POSITION

PRESS NEXT                              Bypass rest'n: N

CHANGE

PRESS CHANGE TO "Y" IF YOU WANT TELEPHONES WHICH ARE BARRED

FROM OUTGOING ACCESS TO ONLY BE ALLOWED TO USE THAT SPEED

DIAL NUMBER CODE.

PRESS NEXT (& PRESS SHOW IF REQD        Speed dial:**

TO PROGRAME NEXT TELEPHONE NUMBER)     (START OF NEXT NUMBER)

TO EXIT FROM PROGRAMMING PRESS THE      End of session

RED/ORANGE "RLS" BUTTON

NORMAL TELEPHONE OPERATION              TIME AND DATE

MERIDIAN NORSTAR ADMIN SECTION NAMES

INSTRUCTIONS FOR PROGRAMMING NAMES ONTO EXTN PHONES

NOTE!

DO NOT PICK UP THE HANDSET DURING PROGRAMMING. ALSO THE

PHONE WILL NOT BE ABLE TO MAKE OR RECEIVE ANY CALLS.

USING A M7310 OR M7324 TERMINAL               DISPLAY

PRESS THE "FEATURE" BUTTON

ENTER  ** 23646 (SPELLS ADMIN)          Password:

RETRY

ENTER 23646 (OR OWN PASSWORD)           1.Sys Speed Dial

PRESS NEXT                              2.Names

PRESS SHOW                              Extn names

PRESS SHOW                              Show extn:_

OR PRESS NEXT                           Line names

AND THEN SHOW TO CHANGE

Line names (01-08 or 001-094)           Show line:_

ENTER EXTN NUMBER OR PRESS NEXT TO

SHOW FIRST EXTN NUMBER i.e. EXTN 21     21:   21

CHANGE

PRESS CHANGE AND TYPE IN NAME USING     21:    XXXXXXX

KEYPAD (UP TO 7 CHARACTERS) PRESS       <---  BKSP  --->

ARROW ON RIGHT OR "GATE" TO MOVE TO

NEXT SPACE (BKSP = BACKSPACE 1 CHARACTER)

PRESS NEXT TO GO TO NEXT EXTN NAME      22:    YYYYYYY

CHANGE

CHANGE NAME IF REQUIRED

TO EXIT FROM PROGRAMMING PRESS THE      End of session

RED/ORANGE "RLS" BUTTON

NORMAL TELEPHONE OPERATION              TIME AND DATE

ON THE M7310 OR M7324 TERMINAL,THE GREY NORSTAR PROGRAMMING

OVERLAY CARD IS PLACED OVER THE TOP 4 BUTTONS BELOW THE "TIME

AND DATE" DISPLAY. (M7324 ON TOP RIGHT HAND SIDE OF TERMINAL)

AS FOLLOWS...

-------------------------------

|  HEADING  |       |    SHOW   |

|   BACK    |       |    NEXT   |

-------------------------------

MERIDIAN NORSTAR ADMIN SECTION TIME AND DATE

INSTRUCTIONS FOR PROGRAMMING TIME & DATE ONTO ALL PHONES

NOTE!

DO NOT PICK UP THE HANDSET DURING PROGRAMMING. ALSO THE

PHONE WILL NOT BE ABLE TO MAKE OR RECEIVE ANY CALLS.

USING A M7310 OR M7324 TERMINAL                               DISPLAY

PRESS THE "FEATURE" BUTTON

ENTER  ** 23646 (SPELLS ADMIN)                           Password:

RETRY

ENTER 23646 (OR OWN PASSWORD)                           1.Sys Speed Dial

PRESS NEXT                                              2.Names

PRESS NEXT                                              3.Time and Date

PRESS SHOW                                              Time  *:**

CHANGE

PRESS CHANGE                                            Hour: **

CHANGE

PRESS CHANGE AND KEY IN HOUR USING

KEYPAD OR GO TO NEXT STEP

PRESS NEXT                                              Minutes: **

CHANGE

PRESS CHANGE AND KEY IN MINUTES

USING KEYPAD OR GO TO NEXT STEP

PRESS NEXT                                              AM or PM

PRESS CHANGE TO ALTER SETTING                                       CHANGE

PRESS NEXT                                              Date 01 Jan 01

CHANGE

PRESS CHANGE                                            Year: **

CHANGE

PRESS CHANGE AND TYPE IN YEAR

USING KEYPAD OR GO TO NEXT STEP

PRESS NEXT                                              Month: **

CHANGE

PRESS CHANGE AND TYPE IN MONTH

USING KEYPAD OR GO TO NEXT STEP

PRESS NEXT                                              Day: **

CHANGE

PRESS CHANGE AND TYPE IN DAY

USING KEYPAD OR GO TO NEXT STEP

PRESS NEXT                                              4. Direct dial

or (4. Restrictions)

TO EXIT FROM PROGRAMMING PRESS THE                      End of session

RED/ORANGE "RLS" BUTTON

NORMAL TELEPHONE OPERATION                              TIME AND DATE